V2.0

Orbiter 
Versatile Point Source LED Light

Orbiter 
Versatile Point Source LED Light

Revision history

| Date | Revision | Changes | Revisor |
| --- | --- | --- | --- |
| 2021-05-31 | V1.0 | First release | AR |
| 2021-07-14 | V1.1 | Technical corrections | AR |
| 2023-09-06 | V2.0 | Additions | BD |

- General

- The luminaire shall be RGBACL LED directional light luminaire with an electronically controlled LED light source especially with the ability to change the optical and mechanical accessories as needed.

- The luminaire shall be capable of providing fully tunable white light from 2,000 to 20,000K and allow precise manipulation of intensity, green-magenta point (full minus green and full plus green), hue and saturation, RGBW control, xy control, gel and source color selection, and built-in effects.

- The precise continuous manipulation of intensity from 0% to 100% is mandatory.

- All functions shall be controllable through USITT DMX 512-A and fully RDM compatible with a feedback channel for reporting, as well as controllable via ArtNet and sACN.

- An on-board control shall be available to control intensity, color temperature, green-magenta point, hue and saturation, RGBW, xy, gel and source selection, and effects. The control panel can be used directly at the fixture or via a connecting cable remote from the fixture.

- The luminaire shall feature a built-in CMRX receiver for wireless lighting communication and control.

- The luminaire shall feature a built-in spectrometer for measurement and reproduction of ambient color.

- The luminaire shall be available as a manual and hanging pole operated version.

- Attaching of yoke and optical accessories shall work without tools.

- Physical

- The luminaire shall be constructed of die casted aluminum, profiles and molded engineering grade fiber-reinforced thermoplastics.

- The body of the fixture shall be available in blue/silver or matt black finish.

- Technical requirements for the hard-light luminaire:

  - The hard light luminaire shall have dimensions not exceeding 305 mm (12”) in length and 330 mm (13”) in height without yoke, 451 mm (17.8”) in length and 670 mm (26.4”) in height with manual yoke, 411 mm (16.2”) in length and 670 mm (26.4”) with pole operated yoke, and 410 mm (16.1”) in width without optics and accessories.

  - Light aperture shall have a dimension of 42 mm (1.66”) (polygon shaped) with a sturdy accessory interface incl. supply voltage and bi-directional digital communication between fixture and accessory.

  - The manual stirrup shall be made of aluminum profile, the pole operated stirrup shall be made of steel, both with a 28 mm (1 1/8”) spigot attached.

  - High strength friction tilt lock shall guarantee secure locking to eliminate any movement or slippage to ensure the luminaire will stay in position.

  - Top handle for carrying of lamphead and plastic foot for allowing the fixture to stand on a flat surface.

  - A tilt range of +/- 90° is required.

  - The beam angle shall range from 15° to 360° with the change of optical accessory.

  - Weight for the luminaire only shall be 11.7 kg (25.8 lb.), manual version shall be 14.2 kg (31.1 lb.) and for the pole operated version 14.9 kg (32.9 lb.).

  - The safety loop attachment shall absorb forces acting on the fixture in the event of a fall and protect the fixture from damage.

- The luminaire shall be equipped with cooling fans.

- The fan noise emission shall not exceed 20 dBA (1m) at any time.

- The LED emitters used in the fixture should be rated for nominal 50,000-hour LED life to 70% intensity with an estimated color shift over lifetime less than 200 K.

- The luminaire shall provide monitoring of the hours in use and the actual temperature.

- Electrical

- The luminaire shall be furnished with an internal power supply for 100 to 250 V~, 50/60 Hz supply voltage.

- The luminaire shall require power from a non-dim source.

- The nominal power consumption shall be 400 W and shall not exceed 500 W at full output.

- Available power cord options shall include but not be limited to:

  - PowerCon TRUE1 TOP cable with bare ends

  - PowerCon TRUE1 TOP cable and Edison connector

  - PowerCon TRUE1 TOP cable and Schuko connector

  - PowerCon TRUE1 TOP cable and Chinese connector

  - PowerCon TRUE1 TOP cable and Japanese connector

- In addition, power through shall be possible via PowerCon TRUE1 TOP power out plug.

- Only integrated light engines that will not emit light in the ultra-violet or the Infrared spectrum are acceptable.

- A control and indicator panel for on-board and remote control shall be available.

- The fixture shall be equipped with an RDM/DMX interface.

- An XLR 5-pin DMX in and XLR 5-pin DMX through shall be implemented.

- Two EtherCon LAN ports shall be implemented, incl. an integrated ethernet switch.

- An XLR 3-Pin port shall be implemented for 48 V DC power input from a regulated external DC source like power station or battery.

- The fixture shall have two USB-A ports, one 1.5 A and one 0.5 A, used for updating the fixture’s internal firmware, adjusting operating parameters and for powering external devices; a USB-C port for service purposes.

- An SD card reader offers an additional option for updating the fixture’s internal firmware or favorite storage.

- An SMA Sync input (TTL) for camera synchronization and external trigger input shall be available.

- An integrated wireless DMX by LumenRadio CRMX (DMX and RDM commands) shall be available.

- An integrated spectrometer for measurement and reproduction of ambient color shall be available.

- Optical

- The optical system shall offer an interchangeable accessory interface with power and bi-directional communication between and fixture and optical accessory. The luminaire should have a hard, point source beam of light and clean shadow rendition with following optical characteristics:

- Aperture dimension with diameter of 42 mm, polygon shaped

- Color rendition index CRI of > 98, TLCI of > 95 and TM30 > 94 at 3,200 to 5,600 K

- Continuously variable correlated color temperature range from 2,000 K – 20,000 K

- Continuously variable green-magenta adjustment

- Full RGBACL color gamut with hue and saturation control

- Digital gel color and source selection

- Calibrated CIE 1931 xy control, with selectable gamuts

- Lighting effects selection

- Color temperature tolerance of 5% (nominal), +/- 1/8 Green-Magenta (nominal)

- The fixture shall be equipped with an optics recognition technology that reads out the information from the mounted optic, while optics status shows on the user interface in real time.

- The manufacturer shall ensure that there will be no differences in the quality of the light field between production batches of the lighting fixtures.

- The fixture should have the following photometric characteristics with open face optics:

@3,200 K

| 1 m / 3.3 ft |  | 3 m / 9.8 ft |  | 5 m / 16.4 ft |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HiCR | HiOut | HiCR | HiOut | HiCR | HiOut |  |  |  |  |
| 15,900 lx | 16,100 lx | 1,780 lx | 1,800 lx | 640 lx | 645 lx | 60° |  |  |
| 41,600 lx | 44,700 lx | 4,620 lx | 4,970 lx | 1,665 lx | 1,790 lx |  | 30° |  |
| 130,000 lx | 138,000 lx | 14,445 lx | 15,330 lx | 5,200 lx | 5,520 lx |  |  | 15° |
| 12,082 fc | 12,825 fc | 1,342 fc | 1,425 fc | 483 fc | 513 fc |  |  |  |
| 3,866 fc | 4,154 fc | 430 fc | 462 fc | 155 fc | 166 fc |  | 30° |  |
| 1,485 fc | 1,490 fc | 164 fc | 167 fc | 59 fc | 60 fc | 60° |  |  |

@5,600 K

| 1 m / 3.3 ft |  | 3 m / 9.8 ft |  | 5 m / 16.4 ft |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HiCR | HiOut | HiCR | HiOut | HiCR | HiOut |  |  |  |  |
| 20,000 lx | 21,000 lx | 2,220 lx | 2,333 lx | 800 lx | 840 lx | 60° |  |  |
| 54,900 lx | 58,000 lx | 6,100 lx | 6,445 lx | 2,195 lx | 2,320 lx |  | 30° |  |
| 162,000 lx | 178,000 lx | 18,000 lx | 19,780 lx | 6,480 lx | 7,120 lx |  |  | 15° |
| 15,056 fc | 16,543 fc | 1,673 fc | 1,838 fc | 602 fc | 662 fc |  |  |  |
| 5,102 fc | 5,390 fc | 567 fc | 599 fc | 204 fc | 216 fc |  | 30° |  |
| 1,859 fc | 1,952 fc | 206 fc | 217 fc | 74 fc | 78 fc | 60° |  |  |

- The fixture should have the following photometrics characteristics with a 15° - 65° Fresnel optic:

@3,200 K

| 1 m / 3.3 ft |  | 3 m / 9.8 ft |  | 5 m / 16.4 ft |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HiCR | HiOut | HiCR | HiOut | HiCR | HiOut |  |  |  |  |
| 6,000 lx | 6,600 lx | 667 lx | 733 lx | 240 lx | 264 lx | 65° |  |  |
| 28,000 lx | 29,500 lx | 3,111 lx | 3,278 lx | 1,120 lx | 1,180 lx |  | 30° |  |
| 80,000 lx | 84,400 lx | 8,889 lx | 9,378 lx | 3,200 lx | 3,376 lx |  |  | 15° |
| 7,432 fc | 7,841 fc | 826 fc | 871 fc | 297 fc | 314 fc |  |  |  |
| 2,601 fc | 2,741 fc | 289 fc | 305 fc | 104 fc | 110 fc |  | 30° |  |
| 557 fc | 613 fc | 62 fc | 68 fc | 22 fc | 25 fc | 65° |  |  |

@5,600 K

| 1 m / 3.3 ft |  | 3 m / 9.8 ft |  | 5 m / 16.4 ft |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HiCR | HiOut | HiCR | HiOut | HiCR | HiOut |  |  |  |  |
| 7,500 lx | 8,200 lx | 833 lx | 911 lx | 300 lx | 328 lx | 60° |  |  |
| 35,200 lx | 38,200 lx | 3,911 lx | 4,244 lx | 1,408 lx | 1,528 lx |  | 30° |  |
| 101,000 lx | 110,000 lx | 11,222 lx | 12,222 lx | 4,040 lx | 4,400 lx |  |  | 15° |
| 9,383 fc | 10,219 fc | 1,043 fc | 1,135 fc | 375 fc | 409 fc |  |  |  |
| 3,270 fc | 3,549 fc | 363 fc | 394 fc | 131 fc | 142 fc |  | 30° |  |
| 697 fc | 762 fc | 77 fc | 85 fc | 28 fc | 30 fc | 60° |  |  |

- The fixture should have the following photometrics characteristics with Projection optics:

@3,200 K

| 3 m / 9.8 ft |  |  | 5 m / 16.4 ft |  | 10 m / 32.8 ft |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HiCR |  | HiOut | HiCR | HiOut | HiCR | HiOut |  |  |  |  |
| 3000 lx |  | 3133 lx | 1080 lx | 1128 lx | 270 lx | 282 lx |  | 35° |  |
| 5878 lx |  | 6200 lx | 2116 lx | 2232 lx | 529 lx | 558 lx |  |  | 25° |
| 546 fc |  | 576 fc | 197 fc | 207 fc | 49 fc | 52 fc |  |  |  |
| 279 fc |  | 291 fc | 100 fc | 105 fc | 25 fc | 56 fc |  | 35° |  |

@5,600 K

| 3 m / 9.8 ft |  | 5 m / 16.4 ft |  | 10 m / 32.8 ft |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HiCR | HiOut | HiCR | HiOut | HiCR | HiOut |  |  |  |  |
| 3,744 lx | 4,078 lx | 1,348 lx | 1,468 lx | 337 lx | 367 lx |  | 35° |  |
| 7,344 lx | 8,000 lx | 2,644 lx | 2,880 lx | 661 lx | 720 lx |  |  | 25° |
| 682 fc | 743 fc | 246 fc | 268 fc | 61 fc | 67 fc |  |  |  |
| 348 fc | 379 fc | 125 fc | 136 fc | 31 fc | 34 fc |  | 35° |  |

- Environmental

- The fixture shall be rated IP20 for dry location use, IP24 with Rain Cover accessory between -90° (down) and +75° (up) tilt angle.

- The fixture shall operate in an ambient temperature range of -20°C (-4°F) to 45°C (113°F).

- Certification

- The fixture shall be compliant with CE standards as well as ENEC and FCC certified.

- The fixture shall be UL LISTED, or equivalent certification, to the UL1573 standard for stage and studio use.

- Operation

- It shall be possible to remote control the luminaire via USITT DMX 512 A.

- The fixture shall be fully E1.20 RDM compatible and equipped with a feedback channel for reporting.

- The fixture shall be fully Art-Net 4 enabled with DMX gateway capability.

- The fixture shall be fully sACN enabled with DMX gateway capability.

- A removable onboard control with 4’’ full color graphical display for intensity and color control, and to access to all options and control possibilities shall be available.

- The fixture shall be accessible via web page when connected to a network and allow changes to fixtures settings, including fixture settings, operational mode, DMX settings and network settings.

- Special modes of operation should be available including, but not limited to:

  - Color Mode: Selects between nine color modes including CCT, HSI, XY Coordinates, RGBACL, Gel Selection, Source Matching, Effects, Cue, and Color Sensor

  - Operational Mode: Selects between max. output (High Output) or best color rendition (High Color). A low noise option for the need quiet operation shall be available too.

  - Tungsten Mode: Mimics the behavior of a conventional Tungsten light source

  - Dimming Curve Selection: Selection of four dimming curves including exponential, linear, logarithmic, and “S” curve

  - Host/Client Mode: Allows for one fixture to control a group of several other ARRI fixtures

  - High Speed Mode: For high speed recording up to 25,000 fps and extreme shutter angles

  - Color Sensor Mode: Reads and reproduces the ambient color surrounding the fixture

  - Lighting Effects: Choose between 15 pre-programmed effects, incl. “Party Effect” that scrolls through hue and saturation

  - Calibrated RGBACL Color Space: generates a calibrated color according to Kodak Pro Photo Color Gamut / ESTA standard E1.54

- Other functions and setting of the fixture should include:

  - Display Setup: Allows for control of the display illumination, brightness, contrast, orientation, and error display setting

  - Flash Drive Functions: Allows for the saving of presets, fixture settings and error logs to an external flash drive like USB stick or SD card

  - Low Battery Warning: Allows user to set a voltage whereby the fixture will display a warning if a DC voltage should fall below

  - DMX Settings: Allows for the adjustment of DMX address, protocol version, and signal loss behavior

  - Network Settings: Allows the adjustment of parameters when the fixture is part of a network

  - Art-Net Settings: Allows for the adjustment of Art-Net net, subnet, universe, merge mode, state, and gateway

  - sACN Settings: Allows for the adjustment of sACN universe

  - IP Settings: Allows for the adjustment of the IP Mode and IP Address with the onboard control panel

  - User shall be able to define and store 246 favorites.

  - Extended Color Control: Allows for modification of the selected color channel

  - Favorites: Allows to program, save and load user defined settings

  - Optics Auto-Adjust: Allows for the adjustment of LED color mixing in order to account for changes caused by optics and accessories.

- The luminaire shall offer nine 8 bit DMX profiles, and nine 16 bit DMX profiles, which can be pre-configured by the user.

- Additionally, any DMX profile shall have the option to add additional channels for specifying color with Extended Color Control.

- Dimming

- The fixture shall allow continuous linear and flicker free dimming from 0% to 100% in an 8 bit mode (0.3922% per step) or 16 bit mode (0.001529% per step).

- The smooth dimming behavior shall be achieved by a combination of high PWM frequency, dynamic frequency adaption and current control.

- In High Speed mode dimming from 100% to 5% shall be possible.

- Accessories

The following accessories shall be available:

- General accessories:

a. Safety cable

b. Mini Noga arm for mounting of Control Panel

- Optical accessories:

- Open Face 15°, 30° and 60° incl. matching 4 or 8-leaf barndoor

- Fresnel with motorized zoom, offering beam angle between 15° - 65°

- 25° and 35° Projection optic that accepts B-size gobos and includes internal shutters for precise shaping of the beam.

- Omnidirectional Dome in four different sizes (S to XL)

- Softboxes in rectangular shape: DoPchoice Snapbag S & M 
and Chimera Light Banks S & M

- Softboxes in octagonal shape: Chimera Octa4 and DoPchoice Octa4

- Glass cover for direct illumination from the light engine

- Other accessories:

- Transporting case

- Soft Bag for Open Face Optics

- Snoot for Open Face

- Rain Cover

- Control Panel

- Skid

- Extension cables for Control Panel

- Yoke for manual and pole operation