V2.0 – Single Page

S30-C 
LED Soft Light

- General

The luminaire shall be a 200 W color mixing RGBW LED panel luminaire with an electronically controlled light source. The luminaire shall be capable of providing fully tunable white light from 2,800 to 10,000K and allow precise manipulation of intensity, green-magenta point (full minus green and full plus green), hue and saturation, RGBW control, xy control, gel and source color selection, and built-in effects. Control shall be possible via on-board control, wired remote control, DMX 512A with RDM capability, and ArtNet or sACN with DMX gateway capabilities. The luminaire shall be available as a manual and hanging pole operated.

- Physical

The luminaire shall be constructed of rugged, aluminum and molded engineering grade fiber-reinforced thermoplastics.

The body of the fixture shall be available in blue/silver or matt black finish.

The luminaire shall have dimensions not exceeding 507 mm (20”) in length, 426 mm (16.8”) in height without spigot, and 133 mm (5.2”) in width.

A tilt range of +/- 90° is required.

- Electrical

The luminaire shall be furnished with an external power supply for 110 to 240 V AC 50/60 Hz supply voltage.

The maximum power consumption shall be 200 W.

An XLR 4-Pin port shall be implemented for battery power input in the range of 23 – 36 V DC

The LED emitters used in the fixture should be rated for nominal 50.000-hour LED life to 70% intensity.

The luminaire shall provide monitoring of the hours in use and the actual temperature.

- Optical

The optical system shall offer an interchangeable diffusion panel with a half peak angle in the range of 72° – 114º.

The luminaire should have soft, even beam of light and clean shadow rendition with following optical characteristics:

Color rendition index CRI of > 95, TLCI of > 91

Color temperature tolerance of +/- 100 K (nominal), +/- 1/8 Green-Magenta (nominal)

The manufacturer shall ensure that there will be no differences in the quality of the light field between production batches of the lighting fixtures.

The luminaire should have the following photometric characteristics:

|  | 3 m / 9.8ft |  | 5 m / 16.4 ft |  | 7 m / 23.0 ft |  | 9 m / 29.5 ft |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3200K | 5600K | 3200K | 5600K | 3200K | 5600K | 3200K | 5600K |
| Standard Diffusion | 670 lx | 782 lx | 241 lx | 282 lx | 123 lx | 144 lx | 74 lx | 87 lx |
|  | 62 fc | 73 fc | 22 fc | 26 fc | 11 fc | 13 fc | 7 fc | 8 fc |

- Environmental

The fixture shall hold the following Certifications & Declarations of Conformity - CE, UKCA, CB, ENEC, cNRTLus, FCC, PSE.

- Operation

The luminaire shall offer ten 8 bit DMX profiles, ten 16 bit DMX profiles, and five coarse/fine DMX profiles, which can be pre-configured by the user.

- Accessories

The following accessories shall be available, but are not limited to:

4-leaf barn-door

Heavy, Lite, Standard, and Intensifier Panels

30º and 60° Honeycomb

SkyBender

4-Chamber eggcrate

Chimera and DoP Choice light banks