V1.0

S360-C 
LED Soft Light

- General

- The luminaire shall be a RGBW LED soft light luminaire with an electronically controlled LED light source, especially with the ability to change the diffusion panel as needed.

- The luminaire shall be capable of providing fully tunable white light from 2,800 to 10,000K and allow precise manipulation of intensity, green-magenta point (full minus green and full plus green), hue and saturation, RGBW control, xy control, gel and source color selection, and built-in effects.

- The precise continuous manipulation of intensity from 0% to 100% is mandatory.

- All functions shall be controllable through USITT DMX 512-A and fully RDM compatible with a feedback channel for reporting, as well as controllable via ArtNet and sACN.

- The luminaire shall feature a built-in CMRX receiver for wireless lighting communication and control.

- An on-board control shall be available to control intensity, color temperature, green-magenta point, hue and saturation, RGBW, xy, gel and source selection, and effects.

- The luminaire shall be available as a carbon fiber and short yoke version.

- Physical

- The luminaire shall be constructed of rugged, aluminum and molded engineering grade fiber-reinforced thermoplastics.

- The body of the fixture shall be available in blue/silver finish.

- Technical requirements for the soft light luminaire:

  - The soft light luminaire shall have dimensions not exceeding 1594 mm (62.8”) in length, 1122 mm (44.2”) in height without spigot, 1237 mm (48.7”) with spigot, and 203 mm (8”) in width.

  - Light aperture shall have a dimension of 1290 mm (50.8”) by 900 mm (35.4”) with a sturdy accessory, barndoor and diffusion slot including a top latch to allow for additional accessories.

  - The stirrup shall be made of carbon fiber with a 28 mm (1 1/8”) spigot.

  - High strength tilt lock shall guaranty secure locking to eliminate any movement or slippage to ensure the luminaire will stay in position.

  - Four plastic handles for carrying of lamphead, and two metal feet for allowing the fixture to stand on a flat surface.

  - A tilt range of +/- 90° is required.

  - The beam angle shall range from 73° – 114º with the changing of diffusion panels.

  - Weight for the carbon fiber yoke version shall be 45 kg (98 lb.) and for the short yoke version 45 kg (98 lb.)

- The luminaire shall be equipped with cooling fans.

- The fan noise emission shall not exceed 20 dBA (1m) at any time.

- The LED emitters used in the fixture should be rated for nominal 50,000-hour LED life to 70% intensity with an estimated color shift over lifetime less than 200 K.

- The luminaire shall provide monitoring of the hours in use and the actual temperature.

- Electrical

- The luminaire shall be furnished with an external power supply for 100 to 240 V~ 50/60Hz supply voltage.

- The luminaire shall require power from a non-dim source.

- The maximum power consumption shall be 1500 W.

- Available power cord options shall include but not be limited to:

  - PowerCon 32A cable with bare ends

  - PowerCon 32A cable and Edison connector

  - PowerCon 32A cable and Schuko connector

  - PowerCon 32A cable and Chinese connector

  - PowerCon 32A cable and Japanese connector

- Only integrated light engines that will not emit light in the ultra-violet or the Infrared spectrum are acceptable.

- A control and indicator panel for on-board control shall be available.

- The fixture shall be equipped with an RDM/DMX interface.

- An XLR 5-pin DMX in and XLR 5-pin DMX through shall be implemented.

- A EtherCon LAN port shall be implemented.

- A locking 4-pin port shall be implemented for 54 V DC power input from the SkyPanel PSU.

- The fixture shall have a USB-A port used for updating the fixture’s internal firmware, adjusting operating parameters, for service purposes, and for powering external devices.

- The fixture shall have a built-in CRMX receiver for wireless lighting communication and control.

- Optical

- The optical system shall offer an interchangeable diffusion panel with a half peak angle in the range of 73° – 114º. The luminaire should have soft, even beam of light and clean shadow rendition with following optical characteristics:

- Aperture Dimension of 1290 mm x 900 mm (50.8” x 35.4”)

- Color rendition index CRI of > 95, TLCI of > 91

- Continuously variable correlated color temperature range from 2,800 K – 10,000 K

- Continuously variable green-magenta adjustment

- Full RGBW color gamut with hue and saturation control

- Calibrated CIE 1931 xy control

- Digital gel color selection

- Lighting effects selection

- Color temperature tolerance of +/- 100 K (nominal), +/- 1/8 Green-Magenta (nominal)

- The manufacturer shall ensure that there will be no differences in the quality of the light field between production batches of the lighting fixtures.

- Fixture should have the following photometric characteristics:

|  | 3 m / 9.8ft |  | 5 m / 16.4 ft |  | 7 m / 23.0 ft |  | 9 m / 29.5 ft |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3200K | 5600K | 3200K | 5600K | 3200K | 5600K | 3200K | 5600K |
| Standard Diffusion | 5878 lx | 6444 lx | 2116 lx | 2320 lx | 1080 lx | 1184 lx | 653 lx | 716 lx |
|  | 546 fc | 599 fc | 197 fc | 216 fc | 100 fc | 110 fc | 61 fc | 67 fc |
| Lite Diffusion | 6022 lx | 6600 lx | 2168 lx | 2376 lx | 1106 lx | 1212 lx | 669 lx | 733 lx |
|  | 559 fc | 613 fc | 202 fc | 221 fc | 102 fc | 112 fc | 62 fc | 68 fc |
| Heavy Diffusion | 4833 lx | 5311 lx | 1740 lx | 1912 lx | 888 lx | 975 lx | 537 lx | 590 lx |
|  | 449 fc | 493 fc | 162 fc | 178 fc | 82 fc | 90 fc | 50 fc | 55 fc |
| Intensifier | 8700 lx | 9544 lx | 3132 lx | 3436 lx | 1598 lx | 1753 lx | 967 lx | 1060 lx |
|  | 808 fc | 887 fc | 291 fc | 319 fc | 148 fc | 162 fc | 90 fc | 99 fc |

- Environmental

- The fixture shall be rated IP 20 for dry location use

- The fixture shall operate in an ambient temperature range of -20°C (-4°F) to 45°C (113°F)

- The fixture shall be in compliance with CE standards as well as ENEC and FCC certified

- The fixture shall be UL LISTED, or equivalent certification, to the UL1573 standard for stage and studio use.

- Operation

- It shall be possible to remote control the luminaire via DMX 512 A.

- The fixture shall be fully RDM compatible and equipped with a feedback channel for reporting.

- The fixture shall be fully Art-Net 3 enabled with DMX gateway capability.

- The fixture shall be fully sACN enabled with DMX gateway capability.

- An onboard control with dot-matrix display for intensity control and to access the menu shall be available.

- Special modes of operation should be available including, but not limited to:

  - Tungsten Mode: Mimics the behavior of a conventional Tungsten light source

  - Low End Dimming Mode: Provides lower light levels with enhanced CCT accuracy and color rendition

  - Dimming Curve Selection: Selection of four dimming curves including exponential, linear, logarithmic, and “S” curve

  - Host/Client Mode: Allows for one fixture to control a group of several other ARRI fixtures.

  - Lighting Effects: 17 different lighting effects, such as “cop car” and “fire”.

  - Extended Color Control: Allows for an additional layer of color adjustment based on color temperature, saturation, +/- red, +/- green, +/- blue, +/- cyan, +/- magenta, +/- yellow.

- Other functions and setting of the lamphead should include:

  - Display Setup: Allows for control of the display illumination, brightness, contrast, Orientation, and error display setting

  - USB Functions: Allows for the saving of presets, fixture settings and error logs to an external USB stick

  - Low Battery Warning: Allows user to set a voltage whereby the lamphead will display a warning if a battery voltage should fall below

  - DMX Settings: Allows for the adjustment of DMX address, protocol, signal loss behavior, and protocol version

  - Art-Net Settings: Allows for the adjustment of Art-Net net, subnet, universe, merge mode, state, and gateway

  - IP Settings: Allows for the adjustment of the IP Mode and IP Address with the onboard control panel

  - CRMX Settings: Allows for turning on and off the CRMX wireless receiver, entering linking keys, and related functions.

- The luminaire shall offer thirteen 8 bit DMX profiles, thirteen 16 bit DMX profiles, and five coarse/fine DMX profiles, which can be pre-configured by the user.

- Additionally, any DMX profile shall have the option to add additional channels for specifying color with Extended Color Control.

- Dimming

- The fixture shall allow continuous linear and flicker free dimming from 0% to 100% in an 8 bit mode (0.3922% per step) or 16 bit mode (0.001529% per step).

- Coarse and fine dimming shall be possible with 2 consecutive DMX channels in the 8 bit mode. The first channel shall allow setting the target value in 256 steps from 0 to 100% output value. The second channel shall allow an additional fine adjustment in 256 steps from 0 to 10% output value.

- Accessories

Following accessories shall be available

1. General accessories:

a. Safety cable

b. Junior pipe clamp

2. Following front end accessories:

c. SkyPanel Remote

e. 30º Honeycomb

f. 60º Honeycomb

g. Heavy, Lite, Standard, and Intensifier Panels

j. Chimera light banks

k. DoP choice light banks

q. 5 and 10 m DC Cable

t. Super Clamp Adapter for SkyPanel PSU

v. Rolling Case