V1.0 – Single Page

S360-C 
LED Soft Light

- General

The luminaire shall be a 1500 W color mixing RGBW LED panel luminaire with an electronically controlled light source. The luminaire shall be capable of providing fully tunable white light from 2,800 to 10,000K and allow precise manipulation of intensity, green-magenta point (full minus green and full plus green), hue and saturation, RGBW control, xy control, gel and source color selection, and built-in effects. Control shall be possible via on-board control, wired remote control, built-in wireless CRMX receiver, DMX 512A with RDM capability, and ArtNet or sACN with DMX gateway capabilities. The luminaire shall be available as a carbon fiber and short yoke version.

- Physical

The luminaire shall be constructed of rugged, aluminum and molded engineering grade fiber-reinforced thermoplastics.

The body of the fixture shall be available in blue/silver finish.

The luminaire shall have dimensions not exceeding 1594 mm (62.8”) in length, 1122 mm (44.2”) in height without spigot, and 203 mm (8”) in width.

A tilt range of +/- 90° is required.

- Electrical

The luminaire shall be furnished with an external power supply for 100 to 240 V AC 50/60 Hz supply voltage.

The maximum power consumption shall be 1500 W.

The LED emitters used in the fixture should be rated for nominal 50.000-hour LED life to 70% intensity.

The luminaire shall provide monitoring of the hours in use and the actual temperature.

- Optical

The optical system shall offer an interchangeable diffusion panel with a half peak angle in the range of 73° – 114º.

The luminaire should have soft, even beam of light and clean shadow rendition with following optical characteristics:

Color rendition index CRI of > 95, TLCI of > 91

Color temperature tolerance of +/- 100 K (nominal), +/- 1/8 Green-Magenta (nominal)

The manufacturer shall ensure that there will be no differences in the quality of the light field between production batches of the lighting fixtures.

The luminaire should have the following photometric characteristics:

|  | 3 m / 9.8ft |  | 5 m / 16.4 ft |  | 7 m / 23.0 ft |  | 9 m / 29.5 ft |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3200K | 5600K | 3200K | 5600K | 3200K | 5600K | 3200K | 5600K |
| Standard Diffusion | 5878 lx | 6444 lx | 2116 lx | 2320 lx | 1080 lx | 1184 lx | 653 lx | 716 lx |
|  | 546 fc | 599 fc | 197 fc | 216 fc | 100 fc | 110 fc | 61 fc | 67 fc |

- Environmental

The fixture shall hold the following Certifications & Declarations of Conformity - CE, UKCA, CB, ENEC, cNRTLus, FCC, PSE.

- Operation

The luminaire shall offer thirteen 8 bit DMX profiles, thirteen 16 bit DMX profiles, and five coarse/fine DMX profiles, which can be pre-configured by the user.

- Accessories

The following accessories shall be available, but are not limited to:

Heavy, Lite, Standard, and Intensifier Panels

30º and 60° Honeycomb

Chimera and DoP Choice light banks